Software Description Specifications

Sculi 2

6E0022

ITM / PME

# Signature

| Author | Date | Name | Contact Information |
| --- | --- | --- | --- |
| Lmbtep0 | 11.05.2012 | Terreaux Patrick | patrick.terreaux@liebherr.com |

| Action* | Date | Issuer Name | Description |
| --- | --- | --- | --- |
| Approved |  |  |  |
| Validated |  |  |  |
| Validated |  |  |  |
| Validated |  |  |  |

*Action: Validated/Not Validated / Approved/Not Approved

# Distribution List

| Name | Department | Contact Information |
| --- | --- | --- |
| Wallmueller Stefan | LMB - Advanced Controls | stefan.wallmueller@liebherr.com |
| Glauser Theodor | LMB - Advanced Controls | theodor.glauser@liebherr.com |
| Rajek Martin | LWN - PME | martin.rajek@liebherr.com |
| Wambacher Roman | LBH - ITM | roman.wambacher@liebherr.com |
| Unterdorfer Thomas | LBH - ITM | thomas.unterdorfer@liebherr.com |
| Kolly Claude | LMB – Customer services | claude.kolly@liebherr.com |
| Burri Markus | LMB – Customer services | markus.burri@liebherr.com |

# Revision History

This section identifies all changes made to this Document between two release versions. The table below summarizes each change including date of revision, implementing author, change description (with REQ number or affected sections) and the Issue number if it exists.

| Ver. no | Date | Issuer | Description (REQ number if available) | Issue No |
| --- | --- | --- | --- | --- |

# Table of Content

1.	Signature	1

2.	Distribution List	2

3.	Revision History	3

4.	Table of Content	4

5.	Purpose	5

6.	Acronyms and abbreviations	5

7.	Overview	5

8.	Functional Requirements	5

8.1.	General features	5

8.2.	Connection	5

8.3.	Information	6

8.4.	Error messages	6

8.5.	Variables	6

8.6.	Oscilloscope	7

9.	User Interface	7

10.	Appendix A: Variable list	8

# Purpose

One of the focuses of Liebherr Machines Bulle SA is the development of his new ECU generation called Lidec 2. Planed for March 2013, 130 engines equipped with Lidec 2 D-Muster will be sent to customer.

This new ECU will no more use the Heinzmann protocol such as for Lidec1, but the diagnostic will be based on the current automotive standards UDS.

The diagnostic software Sculi is not able to support the Lidec 2. According to the current bad architecture and known technology maintenance problems, the ITM acknowledged on 23th Mai 2012 that the best solution in order to have Liebherr diagnostic product for March 2013 is to develop a new product (Sculi 2).

Markus Burri (customer services) has been designed as Key User. His role is to define the priorities and the first features required for March 2013.

The goal of this document is to describe the functionalities of Sculi 2  that have to be present on March 2013 for the diagnostic engines with Lidec 2.

# Acronyms and abbreviations

| Abbr. | Description |
| --- | --- |
| ECU | Electronic Control Unit |
| LIDEC | Liebherr Diesel Electronic Control |
| UDS | Unified Diagnostic Services |
| ODX | Open Diagnostic data eXchange |

# Overview

According to the expertise of Markus Burri, the priorities for the diagnostic of Lidec 2 have been defined:

- Read general information (Lidec software, engine type, …)

- Read active and passive error messages

- Read variables

- Record variables over time (oscilloscope)

# Functional Requirements

## General features

| Req. ID | Description | Jira Nr. | Done |
| --- | --- | --- | --- |
|  | The application must operate on Windows XP and Windows 7 |  |  |
|  | The application must be usable in English, German and French |  |  |
|  | A setup must be available in order to install, uninstall and update the program |  |  |
|  | The application must contain an “about” in order to have information about the current used application version |  |  |

## Connection

| Req. ID | Description | Jira Nr. | Done |
| --- | --- | --- | --- |
|  | The application can auto-detect the available Lidec 2 |  |  |
|  | The application can connect to a Lidec 2 |  |  |
|  | The application can disconnect a Lidec 2 (after connection) |  |  |
|  | The application is able to reconnect to a Lidec 2 if the cable was disconnected and reconnected |  |  |

## Information

| Req. ID | Description | Jira Nr. | Done |
| --- | --- | --- | --- |
|  | The following information concerning the Lidec 2 must be available after a connection: - Identification number (BAAN) - Lidec 2 software version - Specification - Lidec 2 serial number - Engine Type - Motor Serial number |  |  |

## Error messages

| Req. ID | Description | Jira Nr. | Done |
| --- | --- | --- | --- |
|  | The application must allow to read active error messages with the following information: - Message Code - Priority - Name - Description - Time last occurrence - Number of occurrences |  |  |
|  | The application must allow to read passive error messages with the following information: - Message Code - Priority - Name - Description - Time last occurrence - Number of occurrences |  |  |
|  | The application must allow reading additional information of specific messages. The additional information are: - TBD |  |  |
|  | When the message list is visible, the list must be updated every 2 seconds |  |  |

## Variables

| Req. ID | Description | Jira Nr. | Done |
| --- | --- | --- | --- |
|  | The application must allow to read all the variables described in appendix A |  |  |
|  | The application must allow the show the following information for each variables: - Code - Name - Description - Value - Unit - Range - Default Value |  |  |
|  | The application must allow to sort variables per Code, Name, Description, and Unit |  |  |
|  | The application allows to search variables per Code, Name, Description and Unit |  |  |
|  | The value of the visible variables must be updated every 2 seconds |  |  |

## Oscilloscope

| Req. ID | Description | Jira Nr. | Done |
| --- | --- | --- | --- |
|  | The application allows to select and deselect the variables to monitor |  |  |
|  | The application allows to start and stop a record |  |  |
|  | The application allows to define the range of each monitored variable |  |  |
|  | The application allows to define the color of each monitored variable |  |  |
|  | The application allows to change the sampling rate with the following options: - 20 ms - 50 ms - 100 ms |  |  |

# User Interface

# Appendix A: Variable list

- TBD